Ball Peen Hammer

Objective: The student is expected to understand and demonstrate the following operations.

- Practice various lathe operations

- Read work drawings

- Threads and classes of fits

- Assemble machined parts

- Observe the necessity of tolerances

- See how planning a job leads to “Pay Off”

- Tolerance of ±0.005” unless otherwise specified

Requirements:

- Study the drawings first

- Read the steps (Procedures)

- Ask questions

- Wear the proper PPE

- Begin your work

- Work carefully

Revision 1:  7/30/2019

Ball Peen Hammer

Die Cut

Handle Procedures:

- Cut and deburr material ¾" Hot Roll Round (HRR) -6 ¾ " long.

- Center, center drill and face both ends of stock.

- Take stock out 3 ½" and center with live center in center hole. Make sure to set-up the dial indicator as close to the jaws as possible.

- Turn down stock to .620 diameter.

- Bring stock in and center 1" out.

- Face off center hole cut out.

- Set-up compound rest at 45 degrees and cut angle 1/8" long.

- Turn stock around and center with aluminum pads 5 ½" out.

- Turn down the handle body to .500 diameter.

- Set-up the round nose tool and turn the body down to .437 diameter.

- Set-up the turning tool and turn down for the threads down to .372 diameter. (Tolerance of ±0.001”)

- Set-up die to cut 3/8" — 24 threads per inch (TPI).

- Set-up threading tool and cut out threads.

- Set-up lathe and knurling tool to form medium knurls.

- Knurl the 2 ½" long handle and clean.

- Deburr and polish all machined surfaces.

Head Procedures:

Lathe Operations:

- Cut and deburr material 1" H.R.R. — 4" long.

- Chuck up, center 1 ½" out and face one end only.

- Turn down head diameter to .875, 1" long.

- Set-up compound rest to 45 degrees and cut face chamfer 1/8" long.

- Turn around and chuck-up head with aluminum pads ¾" in the jaws.

- Turn down body diameter to .812.

- Turn down ball diameter to .750 as per drawing.

- Set-up the threading tool to cut the grove line as per drawing.

- Set-up the radius tool and turn down the grove on the .812 diameter down to .700.

- Turn down the grove on the .750 diameter down to .600.

- Turn the ball radius by using a flat file at a 45° angle to produce the desired shape. (Ask lab assistant for assistance in this step)

Milling Operations:

- Set-up the parallel bars on the milling vise.

- Set-up soft caps on the vise jaws and set-up hammer head into the vise jaws.

- Set-up the edge finder and locate center of hole on head.

- Set-up the center drill and drill center hole.

- Set-up the Q drill bit in the collet and drill tap hole ½" deep.

- Set-up the ½" end mill and cut counter bore .1 deep.

- Tap the hole with the 3/8"-24 TPI taper tap and then with the bottoming tap.

- Clean out tapped hole and check thread fit with the ball peen hammer handle.

- Set-up on lathe and deburr polish.